Track Mounted Manlift and Scissor Lift 2019-41

Addendum # 1

Revision to IDOT Specification Questionnaire 188-60-15

January 15, 2019

- Addendum #1 is to remove the specified horsepower rating on page 4 of IDOT specification 188-60-15. Please replace the following page to the specification/questionnaire.

Specification 188-60-15

Specifications and Questionnaire for Self-Propelled, Rough Terrain Scissor Lift

The functions of all controls shall be permanently marked (No decals) and encased in a weather tight container.

Complies:  Yes   No

A set of controls mounted in a secure location on the chassis shall be provided.  These controls shall be capable of over-riding the platform controls.

Complies:  Yes   No

Engine:

The engine shall be a current production, 3-cylinder, fuel injected, liquid cooled diesel engine.

Complies:  Yes   No

Component provided:

| Make: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model: |  |  |  |  |  |  |  |  |  |
| Number of cylinders: |  |  |  |  |  |
| Horsepower @ governed RPM: |  |  |  |  |  |  | H.P. |  |
| Rated H.P. (Net) |  |  |  |  | H.P. |  |  |

The engine shall be compliant with all current emission standards in place at the time of this bid and award.

Complies:  Yes   No

Fuel system:

The fuel system shall be diesel and clearly labeled as such.

Complies:  Yes   No

The fuel tank shall be sized properly to manufacturer’s specifications for engine supplied with unit.

| Tank capacity: |  |
| --- | --- |

Cooling System:

Manufacturer’s standard cooling system with extended life antifreeze rated to -30F.

Complies:  Yes   No

Transmission, Axles, & Drive train:

The scissor lift shall be mounted on a severe service, rough terrain chassis, equipped with oscillating axles to provide near level setup in unlevel terrain.  The scissor lift shall be powered by a hydrostatic     all-wheel drive system.

Complies:  Yes   No

The scissor lift shall include a 5º tilt alarm and indicator light to alert the operator of uneven terrain.

Complies:  Yes   No

Page 4 of 7